Fusion 360 API Technical Reference & Parametric Design Adaptations

1. Fusion 360 Thread API — Current State (2025-2026)

1.1 Method Signatures and Current Usage

The Fusion 360 Thread API provides two distinct pathways for creating ThreadInfo objects, each serving different workflow requirements. Understanding their precise signatures, capabilities, and appropriate contexts is essential for robust automation development.

1.1.1 ThreadInfo.create (Class Method)

The ThreadInfo.create method operates as a static class method on the ThreadInfo class itself, offering the most comprehensive and explicit control over thread specification. This approach is particularly valuable when thread characteristics must be precisely defined independent of immediate feature creation context, enabling pre-configuration, validation, and reuse across multiple design scenarios.

Namespace: adsk.fusion.ThreadInfo

Complete Signature:

ThreadInfo.create(isTapered, isInternal, threadType, threadDesignation, threadClass, isRightHanded)

| Parameter | Type | Description | Typical Values |
| --- | --- | --- | --- |
| isTapered | bool | Tapered (conical) vs. parallel (cylindrical) thread form | False for standard machine threads; True for NPT, BSPT pipe threads |
| isInternal | bool | Internal (female) vs. external (male) thread orientation | True for tapped holes, nuts; False for bolts, studs |
| threadType | str | Thread standard family identifier | "M" (ISO metric), "UN" (unified inch), "NPT", "BSP", "ACME" |
| threadDesignation | str | Complete size and pitch specification | "M3x0.5", "M6x1", "1/4-20", "G1/2" |
| threadClass | str | Tolerance class for fit control | "6H" (metric internal), "6g" (metric external), "2A"/"2B" (UN); ignored for tapered threads |
| isRightHanded | bool | Thread helix direction | True (standard right-hand); False (left-hand for specialized applications) |

Return Value: Fully configured ThreadInfo object ready for thread feature or tapped hole creation.

Key Characteristics: The explicit parameter set enables complete specification without contextual inference, making this method ideal for library development, configuration-driven systems, and applications requiring non-standard thread forms (left-hand, tapered, or mixed orientations). The threadClass parameter’s behavior—ignored for tapered threads—reflects fundamental differences in how parallel and tapered thread standards specify tolerances (Autodesk Help) .

1.1.2 createThreadInfo (Instance Method)

The createThreadInfo method provides a streamlined convenience interface through the ThreadFeatures collection, reducing parameter count by inferring certain characteristics from creation context.

Namespace: ThreadFeatures collection object (component.features.threadFeatures)

Complete Signature:

threads.createThreadInfo(isModeled, threadType, threadDesignation, threadClass)

| Parameter | Type | Description | Notes |
| --- | --- | --- | --- |
| isModeled | bool | Critical control: generate actual 3D B-rep geometry (True) or cosmetic annotation only (False) | Positioned first in signature as primary workflow decision |
| threadType | str | Thread standard identifier | Equivalent to ThreadInfo.create |
| threadDesignation | str | Size and pitch specification | Equivalent to ThreadInfo.create |
| threadClass | str | Tolerance class | Equivalent to ThreadInfo.create |

Implicit Defaults and Omissions:

| Omitted Parameter | Default Behavior | Context Dependency |
| --- | --- | --- |
| isTapered | False (parallel threads assumed) | Inferred from threadType if "NPT"/"BSP" specified |
| isInternal | Determined by target geometry | Convex cylindrical face → external; concave hole → internal |
| isRightHanded | True (right-hand threads) | No override mechanism in this method signature |

Return Value: ThreadInfo object with context-dependent configuration, suitable for immediate use in ThreadFeatures.createInput() workflows.

Key Characteristics: The reduced parameter count simplifies common-case implementation, but explicit control over handedness and tapered forms requires fallback to ThreadInfo.create. The prominent isModeled parameter reflects its manufacturing workflow significance—this single boolean determines whether thread geometry will exist in exported manufacturing data (Autodesk Community) .

1.1.3 Current Method Status and Selection Guidance

Both methods remain fully supported in the 2025-2026 API with no deprecation warnings or announced retirement timelines (Autodesk Help) (Autodesk Help) . Their coexistence reflects Autodesk’s API stability commitment, enabling existing codebases to function without modification while accommodating diverse developer preferences.

| Selection Scenario | Recommended Method | Rationale |
| --- | --- | --- |
| Pre-configured thread libraries for multiple applications | ThreadInfo.create | Complete parameter control; context-independent specification |
| Single-feature creation with standard thread forms | createThreadInfo | Reduced complexity; streamlined workflow |
| Left-hand thread requirements | ThreadInfo.create | Only method with explicit isRightHanded parameter |
| Tapered pipe threads with explicit control | ThreadInfo.create | Guaranteed isTapered specification |
| Mixed internal/external thread families | ThreadInfo.create | Explicit orientation prevents context-dependent errors |
| Rapid prototyping with known-good geometry | createThreadInfo | Faster implementation; defaults cover majority of cases |

The architectural relationship between methods suggests createThreadInfo may internally delegate to ThreadInfo.create with default substitution, though this implementation detail is unconfirmed. Performance equivalence has been observed; selection should prioritize code clarity and maintainability over micro-optimization.

1.2 Modeled Threads and STL Export

The isModeled parameter represents one of the most consequential decisions in Fusion 360 thread implementation, with direct impact on manufacturing output quality, file size, and computational performance.

1.2.1 isModeled=True as STL Visibility Prerequisite

Setting isModeled=True is absolutely mandatory for thread geometry to appear in exported STL files or any mesh-based manufacturing format. Cosmetic threads (isModeled=False) exist solely as viewport annotations—texture-mapped decals on cylindrical surfaces that provide visual indication but no actual B-rep geometry (Autodesk) .

The technical distinction is fundamental to Fusion 360’s geometry architecture:

| Aspect | Cosmetic Threads (isModeled=False) | Modeled Threads (isModeled=True) |
| --- | --- | --- |
| Geometric representation | Texture decal on cylindrical face | True helical B-rep surfaces |
| Boolean operations | Not participating | Full participation (union, cut, intersect) |
| Mass properties | Ignored | Included in calculations |
| Interference detection | Not detected | Full collision analysis |
| STL/mesh export | Invisible—smooth cylinder exported | Visible—helical geometry tessellated |
| File size impact | Minimal | Substantial (15–40× increase typical) |
| Regeneration time | Fast | Significant slowdown |

The manufacturing implication is severe: parts with cosmetic threads exported for 3D printing will have no functional thread geometry, requiring post-processing tapping or rendering the part non-functional for threaded assembly. This behavior is consistent across all mesh export formats (STL, OBJ, 3MF) and reflects the architectural separation between design intent annotation and manufacturing geometry (Autodesk) .

1.2.2 Export Quality and Mesh Refinement Considerations

While isModeled=True enables thread visibility, achieving “clean” export requires attention to tessellation parameters. The helical surfaces of modeled threads present challenging geometry for mesh generation: continuous curvature variation, small feature sizes relative to typical tessellation tolerances, and sharp transitions at thread runouts.

Recommended STL Export Settings for Thread-Critical Applications:

| Parameter | Default | Recommended for Threads | Effect |
| --- | --- | --- | --- |
| Deviation (max chord height) | 0.005 in / 0.127 mm | 0.001 in / 0.025 mm | Reduces facet-to-surface approximation error |
| Normal angle tolerance | 10° | 5° | Increases facet density in high-curvature regions |
| Minimum facet size | 0.002 in / 0.051 mm | 0.001 in / 0.025 mm | Prevents excessive subdivision of small features |
| Maximum facet size | 0.500 in / 12.7 mm | 0.100 in / 2.54 mm | Ensures uniform sampling of thread profiles |

For fine-pitch threads (M2–M3, 0.4–0.5 mm pitch), aggressive refinement is essential. A 0.5 mm pitch thread with default 0.127 mm deviation may exhibit visible faceting that compromises thread engagement. Reducing deviation to 0.025 mm typically yields 16–20 facets per thread crest, sufficient for functional prototypes and production parts.

Performance Trade-offs: High refinement dramatically increases polygon count. A fully modeled M6 thread with conservative settings may generate ~10,000 triangles; with production refinement, this can exceed 100,000 triangles. For assemblies with multiple threaded features, this complexity compounds, affecting not only export time but also slicer performance and G-code generation.

1.2.3 Best Practices for Thread-Export Workflows

Based on documented behavior and community experience, the following workflow pattern optimizes thread handling:

| Phase | Thread Mode | Purpose |
| --- | --- | --- |
| Design iteration | Cosmetic (isModeled=False) | Rapid regeneration; responsive parametric updates |
| Design review | Modeled (isModeled=True) | Fit checking; interference validation |
| Manufacturing export | Modeled with custom refinement | Production-quality geometry; STL validation |
| Large assembly work | Cosmetic with simplified proxies | Performance preservation; motion studies |

Configuration-based separation enables efficient management: maintain “Design,” “Review,” and “Manufacturing” configurations with appropriate thread modes, switching via parameter-driven suppression rather than manual feature manipulation.

1.3 Multiple Threads on Single Body — Known Limitations and Constraints

Applying multiple thread features to a single body—whether interior/exterior combinations, multiple pitches, or intersecting thread systems—exposes documented limitations that require defensive implementation.

1.3.1 Body Edge Proximity and Boundary Violations

The most frequently encountered failure mode occurs when thread geometry approaches or exceeds body boundaries, triggering the error: “Error Compute Failed: Thread size is bigger than the body” (Autodesk) .

This constraint reflects the geometric kernel’s requirement that all feature geometry remain strictly within target body bounds, with conservative margins that account for: - Tessellation tolerance effects causing apparent boundary violation - Subsequent fillet/chamfer operations that consume additional material - Manufacturing tolerances requiring material stock beyond nominal dimensions

Practical Implications: Threaded features require substantial material margins. For external M6 threads (6.0 mm major diameter), cylindrical bosses should typically exceed 7.0 mm diameter to ensure reliable feature creation. Internal threads similarly require adequate wall thickness to prevent minor diameter from approaching outer surfaces.

Mitigation Strategies:

| Condition | Solution | Implementation |
| --- | --- | --- |
| Thin-walled boss | Increase boss diameter | boss_OD = thread_OD + 2 × min_wall + safety_margin |
| Deep tapped hole | Reduce thread engagement | Use setDistanceExtent with calculated limit |
| Intersecting features | Reorder feature sequence | Apply threads before shelling/cutting operations |
| Complex geometry | Separate bodies, boolean combine | Create threaded regions as distinct components |

1.3.2 Interior/Exterior Thread Combinations with Different Pitches

The specific scenario of internal and external threads with different pitches on the same body—common in adapter fittings, adjustable mechanisms, and specialized components—presents no explicit API prohibition but encounters practical limitations through geometric interaction and parametric update behavior.

| Configuration | API Behavior | Recommended Practice |
| --- | --- | --- |
| Same pitch, opposite handedness | Supported; no special handling | Verify no geometric intersection |
| Different pitch, same handedness | Supported; independent helix generation | Ensure adequate wall thickness between threads |
| Different pitch, intersecting regions | Computation failure likely | Separate bodies with assembly constraints |
| Tapered + cylindrical combination | Context-dependent success | Apply tapered first; verify with section analysis |

Feature Order Dependency: Thread features apply boolean operations to body B-rep, and sequence affects resulting topology. Empirical testing suggests larger-diameter threads before smaller, and external before internal where geometry permits, though no universal rule guarantees success.

Community-Observed Behavior: Custom thread generation via sweep operations demonstrates that “the more revolutions of the helix the more likely the program will give an error or bog down” (Autodesk Community) . This helical geometry sensitivity extends to native Thread features, suggesting that complex multi-thread scenarios benefit from body separation strategies.

1.3.3 Performance Degradation with Multiple Modeled Threads

Documented performance issues with multiple modeled thread instances in assemblies present significant workflow considerations (Autodesk) . The November 2025 API improvements—achieving 186× speedup for custom graphics operations—do not address underlying B-rep complexity that dominates thread feature regeneration (Autodesk Help) .

Optimization Strategies: - Deferred modeling: Cosmetic threads for design iteration; modeled threads only for final validation/export - Simplified representations: Replace detailed threads with cylindrical proxies for motion studies and interference detection at assembly level - Linked component design: Threaded features in separate, lightly-loaded components - Direct modeling fallback: Use BaseFeature with imported thread geometry for stable references in complex assemblies

2. Fusion 360 Built-in Features — Python API Maturity Assessment

The Fusion 360 Python API exposes substantial modeling capability, with maturity varying significantly across feature categories. This assessment evaluates specific features for direct wrapper exposure, focusing on createInput/add pattern availability.

2.1 Fully Supported: Mature createInput/add Implementations

2.1.1 Loft Features

| Aspect | Details |
| --- | --- |
| Status | Mature, stable, fully supported |
| Pattern | loftFeatures.createInput(operationType) → configure → loftFeatures.add() |
| Core capabilities | Multiple profile sections; guide rails; centerline control; tangent/curvature continuity |
| Configuration | loftSections collection with add(); guideRails collection; isSolid boolean; start/end tangent conditions |

The Loft API demonstrates complete functional parity with the interactive interface. Profile addition via loftSections.add(profile, connectionPoint) enables dynamic section count and explicit alignment control. Guide rail integration through loftInput.guideRails.add(guideRail) supports sophisticated form control for aerodynamic surfaces, ergonomic transitions, and complex mechanical components (Autodesk Help) (Autodesk Help) .

Stability: No significant modifications in recent releases; established pattern suitable for production automation.

2.1.2 Sweep Features (with Guide Rails)

| Aspect | Details |
| --- | --- |
| Status | Mature, fully supported |
| Pattern | sweepFeatures.createInput(path, profile, operation) → configure → sweepFeatures.add() |
| Guide rail control | sweepInput.guideRail = guidePath for orientation/shape control |
| Advanced options | Twist angle (twistAngle); orientation type (Perpendicular, Parallel, etc.); scaling for tapered forms |

The guide rail capability is production-ready, enabling variable-section geometry that would require multiple features or direct modeling in less capable systems. Multiple guide rails can be specified through successive property assignments, with interpolation behavior matching interactive controls (Autodesk Community) (Source) (Source) .

Validation: Community implementations confirm reliable operation for structural tubing, wiring channels, and impeller blade geometry.

2.1.3 Mirror Features

| Aspect | Details |
| --- | --- |
| Status | Fully supported |
| Pattern | mirrorFeatures.createInput() → entity/plane specification → mirrorFeatures.add() |
| Scope | Features, bodies, components |
| Consideration | Default NewBodyFeatureOperation; CombineFeatures required for join behavior |

The Mirror API provides complete symmetric design automation, with automatic dependency tracking for source feature modifications. The noted limitation—absence of direct operation property on input object—requires post-creation boolean operations when unified bodies are desired, but does not prevent functional implementation (Autodesk Community) (Github) .

2.1.4 Linear and Circular Patterns

| Aspect | Linear Pattern | Circular Pattern |
| --- | --- | --- |
| Status | Mature (November 2025 enhancement) | Mature (November 2025 enhancement) |
| Pattern | linearPatternFeatures.createInput() → add() | circularPatternFeatures.createInput() → add() |
| Key parameters | Direction entity; count; distance; spacing type | Axis entity; count; total angle or angle spacing |
| Advanced control | Instance suppression array; bidirectional patterns | Symmetric extension; instance suppression |

The November 2025 release marked critical API completion for pattern features, transitioning from limited addSimple methods to full createInput control (Source) (Autodesk Help) (Autodesk Community) . This enables: dynamic count/spacing via expressions; selective instance suppression for partial arrays; and configuration-driven pattern generation for bolt circles, structural grids, and repetitive features.

2.1.5 Fillet Features (Enhanced 2025-2026)

| Fillet Type | API Availability | Release |
| --- | --- | --- |
| Constant radius | ✓ Mature | Baseline |
| Variable radius | ✓ Mature | Baseline |
| Full round | ✓ Added | Pre-2025 |
| Asymmetric | ✓ Added 2025 | November 2025 |
| Rule-based | ✓ Added 2025 | November 2025 |

The November 2025 enhancement achieved complete fillet API parity with the interactive interface (Autodesk Help) (Autodesk Help) . Asymmetric fillets (AsymmetricFilletEdgeSet, AsymmetricFilletEdgeSetInput) enable distinct radii on opposing faces—critical for casting and molding applications. Rule fillets (RuleFilletFeatureInput, RuleFilletSettings) support automated edge selection based on geometric criteria, substantially reducing automation complexity for fillet-heavy parts.

2.2 Partial or Workaround-Required Features

2.2.1 Coil/Helix Features — Critical API Gap

| Aspect | Details |
| --- | --- |
| Native API | None — no CoilFeatures collection |
| Status | Longstanding community request; acknowledged gap |
| Primary workaround | TemporaryBRepManager.createHelixWire() → PipeFeatures.add() or sweep |
| Community solution | HelixGenerator add-in (Patrick Rainsberry) — empirical plane-based construction |
| Limitation | No parametric editability equivalent to native coil feature; reconstruction required for parameter changes |

The coil/helix gap significantly impacts spring design automation, custom thread generation, and spiral mechanism development. The TemporaryBRepManager workaround generates geometrically correct helices but embeds parameters in construction geometry rather than exposing editable feature parameters (Autodesk Community) (Autodesk Community) (Autodesk Community) (Autodesk Community) .

Recommended wrapper implementation: Encapsulate createHelixWire + PipeFeatures workflow with explicit spring engineering parameters (wire diameter, mean coil diameter, free length, active coils) and reconstruction logic for parametric updates.

2.3 Sheet Metal Features — 2026 Status

2.3.1 Hem Features (January 2026)

| Aspect | Details |
| --- | --- |
| Availability | New — January 2026 release |
| API completeness | Full createInput/add support at launch |
| Hem types | Open, Flat, Rolled, Rope, Teardrop, Double |
| Extensions | Preset support with Design Extension |

The Hem API’s launch with complete programmatic access represents positive new-feature development pattern, contrasting historical API lag. All six hem types are accessible through hemInput.hemType enumeration, with type-specific parameters (length, bend radius, gap) configured through input object properties (Autodesk) .

2.3.2 Flange Features

| Aspect | Details |
| --- | --- |
| Status | Mature, enhanced January 2026 |
| 2026 addition | “Flange to Object” extent type — API-accessible intent-driven termination |
| Pattern | flangeFeatures.createInput(edge) → extent configuration → add() |

The “Flange to Object” enhancement enables adaptive flange design where length automatically adjusts to intersecting geometry, supporting manufacturing-aware workflows without manual measurement (Autodesk) .

2.3.3 Sheet Metal Conversion — Persistent Limitation

| Aspect | Details |
| --- | --- |
| Functionality | Convert non-sheet-metal bodies to sheet metal components |
| API status | UI-only as of February 2026 |
| Community request | Acknowledged; no implementation timeline |
| Workaround | Create sheet metal features directly from sketches; reconstruct rather than convert |

This gap affects imported geometry workflows and direct-to-sheet-metal automation. Recommended architecture: design workflows that create SheetMetalComponent directly with BaseFlangeFeature rather than attempting conversion of solid geometry (Source) .

2.4 Feature API Summary Table

| Feature Category | Specific Feature | createInput/add | Status | Key Notes |
| --- | --- | --- | --- | --- |
| Surface/Solid Creation | Loft | ✓ | Mature | Full profile, guide rail, centerline, continuity control |
|  | Sweep (guide rails) | ✓ | Mature | Multiple rails, twist, orientation options |
|  | Mirror | ✓ | Mature | Feature/body level; CombineFeatures for join |
| Patterning | Linear Pattern | ✓ | Mature (2025+) | Full parameter access; instance suppression |
|  | Circular Pattern | ✓ | Mature (2025+) | Full parameter access; symmetric extension |
| Edge Treatment | Fillet (all types) | ✓ | Mature (2026) | Asymmetric, rule-based added 2025 |
| Helical | Coil/Helix | ✗ | Workaround only | TemporaryBRepManager + pipe/sweep |
| Sheet Metal | Hem | ✓ | New (2026) | All types; presets with Design Extension |
|  | Flange | ✓ | Mature, enhanced 2026 | “Flange to Object” intent-driven |
|  | Convert to Sheet Metal | ✗ | UI-only | Requested; no timeline |

3. TinkerCAD Community Shape Generators — JS Architecture and Fusion 360 Adaptation

The TinkerCAD platform’s community shape generators represent a significant corpus of user-created parametric design tools, with architectural patterns that remain directly applicable to modern CAD automation despite the platform’s evolution. Understanding these patterns enables effective translation to Fusion 360’s more capable environment.

3.1 Original JS-Based Generator Structure

TinkerCAD generators operated within a constrained but elegant JavaScript sandbox, emphasizing accessibility for non-programmer creators while enabling sophisticated geometric generation through constructive solid geometry (CSG) operations.

3.1.1 Parameter Definition Pattern — Declarative Specification

The fundamental parameter interface used a declarative array structure assigned to a global params variable, with each parameter object specifying complete UI and data behavior:

var params = [
  {
    id: "baseWidth",
    displayName: "Base Width",
    type: "number",
    rangeMin: 10,
    rangeMax: 100,
    default: 50,
    step: 1
  },
  {
    id: "mountingStyle",
    displayName: "Mounting Style", 
    type: "list",
    options: ["through-hole", "surface-mount", "press-fit"],
    default: "through-hole"
  }
];

Standard Fields and Their Functions:

| Field | Purpose | UI Impact |
| --- | --- | --- |
| id | Machine-readable identifier; property name in process() function | None (internal) |
| displayName/label | Human-readable label in parameter panel | Dialog control label |
| type | Data type and validation | Control selection: slider, dropdown, checkbox, text |
| rangeMin/rangeMax | Valid value bounds | Slider limits; input validation |
| default | Initial value; reset target | Control initial state |
| step (optional) | Discrete increment for numeric inputs | Spinner arrow increment; slider snap |

This declarative approach eliminated imperative UI construction, enabling rapid generator development without interface programming. The TinkerCAD infrastructure automatically rendered appropriate controls, validated inputs, and coerced values to specified types. Parameter arrays also served as implicit documentation, explicitly specifying valid ranges and defaults for human readers (Through the Interface) .

3.1.2 CSG Operation Composition — Functional Geometry Generation

Generator execution followed a pure functional pattern: the process(params) function transformed parameter values into CSG solid geometry through composed operations, with the platform handling mesh generation and rendering.

Core CSG Operations:

| Operation | Method | Algebraic Equivalent |
| --- | --- | --- |
| Union | solid1.union(solid2) or Solid.union([solid1, solid2, ...]) | A ∪ B |
| Difference/Subtraction | solid1.subtract(solid2) | A B |
| Intersection | solid1.intersect(solid2) | A ∩ B |

Primitive Generators:

| Function | Signature | Use Case |
| --- | --- | --- |
| makeCylinder() | makeCylinder(radius, height, [center]) | Shafts, holes, rounded features |
| makeCube() | makeCube(width, depth, height, [center]) | Prismatic forms, block structures |
| makeSphere() | makeSphere(radius, [center]) | Ball joints, domed features |
| makeTorus() | makeTorus(majorRadius, minorRadius, [center]) | O-ring grooves, decorative elements |

Transformation Chaining: All solids supported transformation methods returning new instances:

var positionedHole = makeCylinder(2, 10)
  .translate([x_pos, y_pos, z_pos])
  .rotateX(90)
  .scale([1, 1, depth/10]);

The immutable, functional pattern enabled predictable composition: complex geometry built through variable assignment and stepwise refinement, with each intermediate state available for inspection and reuse. No side effects or global state modifications meant parameter changes triggered complete, deterministic regeneration (Through the Interface) .

3.1.3 Advanced Construction — 2D Path Extrusion

Beyond primitive solids, TinkerCAD supported 2D profile construction with extrusion:

var path = new Path2D();
path.moveTo(0, 0);
path.lineTo(width, 0);
path.lineTo(width, height);
path.lineTo(0, height);
path.lineTo(0, 0);

var solid = Solid.extrude([path], extrusionHeight);

The grid-of-columns generator exemplified this pattern: nested loops constructed positioned square paths, collected into an array, and extruded as unified geometry with final transformation for proper orientation (Through the Interface) .

3.1.4 Archived Example: Truncated Pyramid Generator

A representative community generator implemented frustum geometry through parameter-driven construction:

| Parameter | Type | Range | Function |
| --- | --- | --- | --- |
| baseWidth | number | 5–100 mm | Bottom square dimension |
| topWidth | number | 1–95 mm | Upper square dimension (constrained < base) |
| height | number | 2–100 mm | Vertical extent |

Implementation Strategy: The frustum could be constructed through multiple valid CSG approaches—scaled pyramid subtraction, direct vertex enumeration, or loft-like profile interpolation—demonstrating generator author flexibility within the parameter contract. The specific implementation choice was encapsulated; users experienced consistent parameter-driven behavior regardless of internal geometry construction (Github) .

3.2 Adaptation for Fusion 360 Parametric Part Catalog

Translating TinkerCAD’s proven patterns to Fusion 360 requires mapping the simplified CSG paradigm to Fusion’s feature-based parametric system, with strategic decisions about editability preservation and performance optimization.

3.2.1 Parameter Mapping — TinkerCAD to Fusion 360 Translation

| TinkerCAD Concept | Fusion 360 Equivalent | Implementation Notes |
| --- | --- | --- |
| params array | Design.userParameters collection | design.userParameters.add(name, value, units, comment) |
| rangeMin/rangeMax | Expression-based bounds or custom validation | Fusion parameters lack native range enforcement; implement via expressions or event handlers |
| default | Initial ValueInput | Can be expression referencing other parameters |
| type: "number" | ValueInput with unit specification | Automatic mm/inch conversion |
| type: "list" | Configuration tables or string parameter with validation | Configuration tables preferred for variant families |
| step | UI spinner increment | FloatSpinnerCommandInput.setIncrement() |

Enhanced Capabilities in Fusion: The translation enables significant functional expansion unavailable in TinkerCAD:

- Expression-driven relationships: baseWidth = insert_OD + 2 * wall_thickness — parameters derive from functional requirements rather than fixed values

- Unit awareness: Automatic conversion between measurement systems

- Configuration variants: Predefined size families (M2/M3/M4/M5/M6 complete sets) through configuration tables

- External data linkage: Excel/CSV-driven parameter tables for manufacturing standards

3.2.2 CSG → Fusion API Translation — Boolean Operation Mapping

| CSG Operation | Fusion Feature-Based | Fusion Direct (Temporary B-Rep) |
| --- | --- | --- |
| union() | combineFeatures.add(target, tools, JoinFeatureOperation) | tempBrep.booleanOperation(target, tool, UnionBooleanType) |
| subtract()/difference() | combineFeatures.add(target, tools, CutFeatureOperation) | tempBrep.booleanOperation(target, tool, DifferenceBooleanType) |
| intersect() | combineFeatures.add(target, tools, IntersectFeatureOperation) | tempBrep.booleanOperation(target, tool, IntersectionBooleanType) |
| makeCylinder() | Sketch circle → extrudeFeatures.addSimple() | tempBrep.createCylinderOrCone() |
| makeCube() | Sketch rectangle → extrudeFeatures.addSimple() | tempBrep.createBox() |
| translate()/rotate() | moveFeatures.add() with transform | Inherent in construction or tempBrep.transform() |

3.2.3 Feature-Based vs. Direct Modeling Approaches

| Aspect | Feature-Based (Parametric History) | Direct Modeling (Temporary B-Rep) |
| --- | --- | --- |
| Editability | Complete — any feature modifiable via timeline | Limited — geometry as single opaque block |
| Design intent | Explicit — feature relationships documented | Implicit — reconstruction required for changes |
| Performance | Degrades with complex feature trees | Fast — minimal timeline overhead |
| Implementation | Higher complexity; sketch-profile-feature indirection | Lower complexity; immediate geometry |
| Best for | User-facing parameters; design exploration | Internal details; large patterns; final features |

Hybrid Recommendation: Use feature-based for primary design intent (overall dimensions, fastener selections, critical interfaces) and direct modeling for internal details (knurl patterns, draft angles, fillet chains, repetitive features). This balances editability and performance while maintaining workflow clarity.

3.3 Community-Driven Parameterization Patterns

Analysis of high-usage TinkerCAD generators reveals consistent categories of user needs that should inform Fusion 360 catalog prioritization.

3.3.1 High-Demand Parameter Categories

| Category | Generator Examples | Fusion 360 Enhancement |
| --- | --- | --- |
| Fastener interfaces | Threaded inserts, nut traps, screw bosses, clearance holes | Brand-specific dimension tables (CNC Kitchen, Ruthex, McMaster); material-specific design guides |
| Structural elements | Brackets, gussets, living hinges, reinforcement ribs | Load-rated sizing with FEA integration; material flexural modulus for living hinges |
| Enclosure features | Snap fits, vent patterns, mounting ears, cable management | IP rating compliance checking; stress concentration analysis |
| Mechanical interfaces | Bearing seats, shaft couplers, pulley profiles, gear blanks | ISO standard integration; tolerance-aware parameterization |

3.3.2 Fusion 360 Catalog Implementation Strategy

Feature Script Modularization: Encapsulate common patterns as reusable modules with standardized interfaces:

class ThreadedInsertBoss:
    def create(self, thread_size, brand, length_type, boss_style):
        dimensions = self._lookup_dimensions(thread_size, brand, length_type)
        # Construct geometry via feature-based or direct modeling
        # Return feature reference for downstream operations

Standard Parameter Naming Conventions:

| Pattern | Example | Purpose |
| --- | --- | --- |
| Dimensional prefix | insert_OD, boss_wall_min, hole_diameter | Cross-script consistency; predictable interfaces |
| Functional suffix | clearance_loose, clearance_tight, clearance_press | Fit type specification |
| Scope indicator | body_, feature_, clearance_ | Context clarity |

Validation Functions: Embed design rule checking to prevent common errors:

def validate_boss_geometry(insert_spec, boss_params):
    required_wall = insert_spec['min_wall']
    actual_wall = (boss_params['boss_diameter'] - insert_spec['insert_OD']) / 2
    
    if actual_wall < required_wall:
        raise ValueError(f"Wall thickness {actual_wall:.2f}mm below minimum {required_wall}mm")
    
    if boss_params['hole_depth'] < insert_spec['length']:
        raise Warning(f"Hole depth may not accommodate full insert length")

Preset Libraries: Capture brand-specific configurations as selectable variants, enabling single-parameter selection that populates complete dimension sets.

4. Heat-Set Insert Dimensions — M2–M6 Brand Comparison

Heat-set threaded inserts provide robust metal threads in 3D-printed thermoplastic parts, with dimensional precision critical for installation success and mechanical performance. Specifications vary across manufacturers, with significant implications for parametric design automation.

4.1 Dimensional Specification Framework

Five measurements govern insert integration and surrounding boss design:

| Parameter | Symbol | Definition | Design Impact |
| --- | --- | --- | --- |
| Insert Outer Diameter | ⌀ D1 | Maximum knurled body diameter | Determines required boss OD; influences pull-out resistance |
| Centre Tip Diameter | ⌀ D2 | Reduced diameter at insertion end | Facilitates alignment; reduces initial insertion force |
| Recommended Hole Diameter | ⌀ D3 | Target bore for thermal/ultrasonic installation | Balances interference fit against installation force |
| Overall Length | L | Total axial insert dimension | Determines minimum boss height; thread engagement depth |
| Minimum Wall Thickness | W | Required radial plastic surrounding insert | Prevents boss cracking under torque and tensile loads |

Derived Design Parameters: - Minimum Boss OD: D1 + 2 × W - Recommended Hole Depth (blind): L + 0.5–1.0 mm (thermal expansion relief) - Recommended Hole Depth (through): L + 1.0 mm (debris clearance)

4.2 Brand-Specific Specifications

4.2.1 CNC Kitchen (Germany) — Complete Documented Series

CNC Kitchen inserts are specifically engineered for 3D printing applications, with extensive community testing and documentation (3DJake International) :

| Thread | Length L (mm) | Insert OD ⌀ D1 (mm) | Centre Tip ⌀ D2 (mm) | Hole ⌀ D3 (mm) | Min Wall W (mm) | Min Boss OD (mm) |
| --- | --- | --- | --- | --- | --- | --- |
| M2 | 3.0 | 3.6 | 3.1 | 3.2 | 1.3 | 6.2 |
| M2.5 | 4.0 | 4.6 | 3.9 | 4.0 | 1.6 | 7.8 |
| M3 | 5.7 | 4.6 | 3.9 | 4.0 | 1.6 | 7.8 |
| M3 Short | 3.0 | 4.6 | 3.9 | 4.0 | 1.6 | 7.8 |
| M3 Voron | 4.0 | 5.0 | 4.25 | 4.4 | 1.3 | 7.6 |
| M4 | 8.1 | 6.3 | 5.5 | 5.6 | 2.1 | 10.5 |
| M4 Short | 4.0 | 6.3 | 5.5 | 5.6 | 2.1 | 10.5 |
| M5 | 9.5 | 7.1 | 6.3 | 6.4 | 2.6 | 12.3 |
| M5 Short | 5.8 | 7.1 | 6.3 | 6.4 | 2.6 | 12.3 |
| M6 | 12.7 | 8.7 | 7.9 | 8.0 | 3.3 | 15.3 |

Notable Design Decisions: - M3 reuses M2.5’s 4.6 mm OD — diameter standardization for tooling efficiency, with length differentiation for application flexibility - M3 Voron variant — enhanced 5.0 mm OD for high-vibration 3D printer motion systems, with reduced wall requirement (1.3 mm) reflecting application-specific optimization - Consistent D2/D3 relationship: Centre tip 0.1–0.7 mm smaller than hole diameter, providing lead-in without compromising interference fit - Wall thickness scaling: W/D1 ratio ~0.35–0.38 across range, maintaining structural integrity without excessive material

4.2.2 Ruthex (Germany) — Industrial-Grade Alternative

Ruthex specifications demonstrate substantial dimensional convergence with CNC Kitchen, suggesting industry-wide optimization for 3D printing compatibility (3DJake International) (3DJake International) (ruthex) :

| Thread | Length L (mm) | Insert OD ⌀ D1 (mm) | Centre Tip ⌀ D2 (mm) | Hole ⌀ D3 (mm) | Min Wall W (mm) | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| M2 | 4.0 | 3.6 | 3.1 | 3.2 | 1.3 | Longer than CNC Kitchen (3.0 mm) |
| M3 | 5.7 | 4.6 | 3.9 | 4.0 | 1.6 | Identical to CNC Kitchen |
| M4 | 8.1 | 6.3 | 5.5 | 5.6 | 2.1 | Identical to CNC Kitchen |
| M5 | 9.5 | 7.1 | 6.3 | 6.4 | 2.6 | Identical to CNC Kitchen |
| M6 | 12.7 | 8.7 | 7.9 | 8.0 | 3.3 | Inferred from pattern match |

Key Differentiator: M2 length 33% longer than CNC Kitchen (4.0 mm vs. 3.0 mm) — enhanced retention for applications where short inserts experience pull-out failures, at cost of deeper boss requirement.

Ruthex drill bit set specifications confirm dimensional alignment: 3.2 mm, 4.0 mm, 5.6 mm, 6.4 mm, 8.0 mm for M2/M2.5, M3, M4, M5, M6 respectively — matching CNC Kitchen D3 values exactly (3DJake International) (ruthex) .

4.2.3 McMaster-Carr (USA) — Industrial Supply Positioning

| Aspect | Details |
| --- | --- |
| Series | 94180A (primary industrial line) |
| Market position | Industrial quality; tighter tolerances than consumer alternatives |
| Documentation status | Detailed M2–M6 dimensional table not recovered from research |
| Known characteristic | Consistent dimensions per community characterization; quality control infrastructure |
| Recommendation | Direct catalog consultation for exact specifications; consider for applications requiring supplier certification |

The absence of published dimensional standards creates practical challenges for automated parametric workflows. Designers must implement empirical measurement protocols or specify alternative brands with documented specifications for reliable automation.

4.3 Comparative Analysis and Cross-Brand Implications

| Comparison Dimension | Finding | Design Impact |
| --- | --- | --- |
| CNC Kitchen vs. Ruthex M3–M6 | Near-identical dimensions | Interchangeable boss design; supply chain flexibility |
| M2 length variation | Ruthex 33% longer | Evaluate pull-out requirements; specify brand explicitly if critical |
| M3 Voron variant | CNC Kitchen exclusive | High-vibration applications; reduced wall requirement |
| Hole diameter convergence | Identical D3 across brands | Standard drill bit set compatibility |
| Wall thickness consistency | ~35–38% of D1 across brands | Predictable boss sizing rules |

Parametric Design Recommendation: Implement CNC Kitchen as primary specification with Ruthex as validated alternative for M3–M6, with explicit brand parameter enabling rapid substitution. McMaster-Carr reserved for applications requiring industrial supplier relationships with direct specification verification.

4.4 Design Implementation Notes

4.4.1 Boss Geometry Calculations

Example — M3 CNC Kitchen Standard Insert:

| Calculation | Formula | Result |
| --- | --- | --- |
| Minimum boss OD | 4.6 + (2 × 1.6) | 7.8 mm |
| Practical boss OD (with 10% margin) | 7.8 × 1.1 | 8.6 mm |
| Minimum boss height (blind hole) | 5.7 + 0.5 | 6.2 mm |
| Recommended boss height | 5.7 + 1.0 | 7.7 mm |

The 10–20% margin on minimum boss OD accommodates FDM print tolerance (±0.2 mm typical), layer adhesion anisotropy, and thermal stress during installation.

4.4.2 Material-Specific Adjustments

| Material | Hole Diameter Adjustment | Rationale |
| --- | --- | --- |
| PLA (brittle) | D3 + 0.1 mm | Reduced insertion stress; prevent boss cracking |
| ABS/ASA | D3 as specified | Standard specification optimized for these materials |
| PETG (ductile) | D3 − 0.05 mm | Enhanced interference for improved retention |
| Nylon (high shrinkage) | D3 − 0.1 mm | Compensate for greater thermal contraction |
| Polycarbonate | D3 − 0.1 mm | Similar to nylon; high thermal expansion |

4.4.3 Fusion 360 Parametric Implementation

User Parameter Structure:

| Parameter | Type | Function |
| --- | --- | --- |
| insert_thread | String | "M2", "M2.5", "M3", "M4", "M5", "M6" |
| insert_brand | String | "CNC_Kitchen", "Ruthex", "McMaster" |
| insert_length_type | String | "Standard", "Short" (where available) |
| insert_safety_factor | Number | 1.0–1.5 multiplier on minimum dimensions |

Expression-Driven Dimensions:

insert_OD = lookup(insert_thread, insert_brand, insert_length_type; table_D1)
insert_length = lookup(insert_thread, insert_brand, insert_length_type; table_L)
insert_min_wall = lookup(insert_thread, insert_brand, insert_length_type; table_W)

boss_OD = (insert_OD + 2 * insert_min_wall) * insert_safety_factor
hole_diameter = lookup(insert_thread, insert_brand; table_D3)
hole_depth = insert_length + 0.75  // Mid-range thermal relief

Validation Architecture: Implement geometric sanity checks via custom feature scripts or parameter update event handlers — minimum wall verification, hole depth adequacy, adjacent feature interference detection.